# Circuit Documentation

## Summary

This circuit is a complex system that integrates various components, including buttons, LEDs, servos, a speaker, a DFPlayer MINI, and an ESP32 microcontroller. The circuit is powered by a power bank through a DC-DC converter, which provides multiple voltage outputs to power different components. The ESP32 microcontroller is used to control the servos and read inputs from buttons, while the DFPlayer MINI is used to play audio through a speaker. The PCA9685 PWM driver is used to control multiple servos, and the circuit also includes ARGB LED strips for visual effects.

## Component List

- Button 12x12

  - Description: A simple push button used for user input.

  - Pins: VCC, OUT, GND

- 12V ARGB LED Strip

  - Description: An addressable RGB LED strip for visual effects.

  - Pins: 12V, GND, DI

- Speaker

  - Description: A speaker for audio output.

  - Pins: VCC, GND

- DFPlayer MINI

  - Description: A small MP3 player module for audio playback.

  - Pins: VCC, BUSY, RX, USB -, TX, USB +, DAC_R, K2/ADC_KEY, DAC_L, K1/ADC_KEY, SPK1, IO 2, GND, SPK2, IO 1

- Servo1

  - Description: A servo motor for mechanical movement.

  - Pins: - Vcc, + Vcc, Signal

- PCA9685

  - Description: A PWM driver for controlling multiple servos.

  - Pins: GND, V+, OE, SCL, SDA, VCC, PWM0-PWM15, V+0-V+15, GND0-GND15

- Resistor

  - Description: A resistor used for current limiting.

  - Properties: 470 Ohms, 1000 Ohms

  - Pins: pin1, pin2

- Electrolytic Capacitor

  - Description: A capacitor for filtering and smoothing voltage.

  - Properties: 1µF, 2.2µF

  - Pins: -, +

- DC-DC 12V to 3.3V 5V 12V Power Module

  - Description: A power module for voltage conversion.

  - Pins: 5vout, Gnd, 3v3 out, DCin out, DCin +, DCin -

- ESP32 Type C

  - Description: A microcontroller for processing and control.

  - Pins: 3V3, GND, D15, D2, D4, D16, D17, D5, D18, D19, D21, RX, TX, D22, D23, VIN, D13, D12, D14, D27, D26, D25, D33, D32, D35, D34, VN, VP, EN

- Powerbank (20000mAh)

  - Description: A portable power source.

  - Pins: USB C input, USB C output

## Wiring Details

### Button 12x12

- VCC connected to 5vout of the DC-DC Power Module.

- OUT connected to various GPIO pins (D4, D16, D17, D5, D18) of the ESP32.

- GND connected to Gnd of the DC-DC Power Module.

### 12V ARGB LED Strip

- 12V connected to DCin out of the DC-DC Power Module.

- GND connected to Gnd of the DC-DC Power Module.

- DI connected through a 470 Ohm resistor to D15 of the ESP32.

### Speaker

- VCC connected to SPK1 of the DFPlayer MINI.

- GND connected to SPK2 of the DFPlayer MINI.

### DFPlayer MINI

- VCC connected to 3v3 out of the DC-DC Power Module.

- GND connected to Gnd of the DC-DC Power Module.

- TX connected through a 1000 Ohm resistor to D19 of the ESP32.

- RX connected to D21 of the ESP32.

### Servo1

- Signal connected to PWM outputs (PWM0-PWM7) of the PCA9685.

- + Vcc connected to V+ outputs (V+0-V+7) of the PCA9685.

- - Vcc connected to GND outputs (GND0-GND7) of the PCA9685.

### PCA9685

- SCL connected to D13 of the ESP32.

- SDA connected to D12 of the ESP32.

- V+ connected to DCin out of the DC-DC Power Module.

- GND connected to Gnd of the DC-DC Power Module.

### Resistor

- 470 Ohms: Used between DI of the ARGB LED Strip and D15 of the ESP32.

- 1000 Ohms: Used between TX of the DFPlayer MINI and D19 of the ESP32.

### Electrolytic Capacitor

- 1µF: Connected across 12V and GND of the ARGB LED Strip.

- 2.2µF: Connected across 12V and GND of the ARGB LED Strip.

### DC-DC 12V to 3.3V 5V 12V Power Module

- DCin + connected to USB C input of the Powerbank.

- DCin - connected to USB C output of the Powerbank.

- 5vout and 3v3 out used to power various components.

### ESP32 Type C

- D4, D16, D17, D5, D18 connected to OUT of various buttons.

- D15 connected to DI of the ARGB LED Strip through a resistor.

- D19 connected to TX of the DFPlayer MINI through a resistor.

- D21 connected to RX of the DFPlayer MINI.

- GND connected to Gnd of the DC-DC Power Module.

### Powerbank (20000mAh)

- USB C input connected to DCin + of the DC-DC Power Module.

- USB C output connected to DCin - of the DC-DC Power Module.

## Code

There is no code provided for this circuit. The ESP32 microcontroller is expected to be programmed to handle button inputs, control the servos via the PCA9685, and manage audio playback through the DFPlayer MINI. The ARGB LED strips can be controlled using appropriate libraries for addressable LEDs.